KENYA PORTS AUTHORITY (KPA)

Enterprise Network Design & Implementation

Phase 1 & Phase 2 — Complete Configuration Document

Cisco Packet Tracer Simulation

VLAN Segmentation  |  OSPF  |  HSRP  |  EtherChannel  |  DHCP & Relay  |  BGP  |  WLC & 802.1X Wireless

# 1. Introduction

## 1.1 Background of the Organization

Kenya Ports Authority (KPA) is a government agency responsible for managing and operating seaports within Kenya. Core responsibilities include cargo handling, vessel management, port security, financial operations, logistics coordination, surveillance monitoring, and administrative services.

Due to the critical nature of port operations, the organization relies on real-time digital communication, data exchange, monitoring systems, and secure financial transactions. A robust, scalable, and highly available enterprise network infrastructure is therefore required to support daily operations and future expansion.

The network must ensure:

- Continuous availability of all operational services

- Secure and isolated communication between departments

- Reliable CCTV surveillance monitoring across port facilities

- Wireless mobility support across all designated port areas

- High-speed data exchange for logistics and cargo operations

- Centralized management of wireless infrastructure

## 1.2 Problem Statement

Traditional flat network architectures are not suitable for large enterprise environments such as port operations. As the number of devices, applications, and departments grows, flat networks introduce several operational challenges:

- Excessive broadcast traffic degrading network performance

- Poor scalability as the organization expands

- Increased network congestion causing service disruptions

- Limited security controls between departments

- Single points of failure leading to network outages

- Difficult troubleshooting and fault isolation

To address these limitations, a structured enterprise network architecture based on the three-tier hierarchical model has been adopted to improve performance, reliability, security, and scalability.

## 1.3 Project Objectives

The primary objective is to design, simulate, and implement a scalable, secure, and highly available enterprise network infrastructure for Kenya Ports Authority using Cisco Packet Tracer. The project specifically aims to:

- Implement a three-tier Hierarchical Network Design (Core, Distribution, Access layers)

- Provide VLAN segmentation for departmental isolation and security

- Implement structured IP addressing and subnetting per department

- Enable inter-VLAN routing at the Distribution layer via SVIs

- Provide full network redundancy to eliminate single points of failure

- Deploy centralized wireless management using a Wireless LAN Controller (WLC) with 802.1X/WPA2-Enterprise

- Implement OSPF Area 0 for internal routing and eBGP for internet connectivity

- Configure DHCP server and relay for centralized IP address management

- Support future scalability and departmental expansion

2. Network Architecture

The KPA network is designed using the Cisco three-tier hierarchical model, separating the network into Core, Distribution, and Access layers.

## 2.1 Core Layer

The Core layer consists of two Cisco 3650 Layer 3 switches operating purely as routed devices. All interfaces are Layer 3 routed ports — no trunk ports, no SVIs for user VLANs, no VLAN database. Responsibilities:

- High-speed packet forwarding between Distribution layer and Border Router

- Dual uplinks to the Border Router for redundancy

- Running OSPF Area 0 to exchange routing information with all internal devices

- EtherChannel (Port-Channel 1) between Core SW1 and Core SW2 via LACP active/active for link redundancy and bandwidth aggregation

## 2.2 Distribution Layer

The Distribution layer consists of two Cisco 3650 Layer 3 switches responsible for all inter-VLAN routing via SVIs. Key functions:

- Hosting all VLAN SVIs and acting as the default gateway for all end devices

- Running HSRP (HSRPv2) for gateway redundancy — virtual IP (.1 or last usable) per VLAN

- Acting as the STP Root Bridge for designated VLANs under Rapid PVST+

- Running OSPF Area 0 and advertising all VLAN subnets

- Forwarding DHCP requests to central DHCP server via ip helper-address

- Connecting to all six Access switches via trunk links

- Phase 2: Dist SW1 Gi1/0/6 converted from routed port to trunk for WLC 3504

## 2.3 Access Layer

Six Cisco 3650 switches, each serving one or more departments. Access switches connect end devices via FastEthernet access ports and uplink to Distribution switches via GigabitEthernet trunk ports. PortFast and BPDUGuard are enabled on all access ports.

Mapped Image

## 2.4 Physical Connectivity Summary

| Device A | Port A | Device B | Port B | Network / Purpose |
| --- | --- | --- | --- | --- |
| ISP Router | Gig0/0 | Border Router | Fa0/0 | 203.0.113.0/30 — Internet link |
| Border Router | Fa0/1 | Core Switch 1 | Gi1/0/1 | 10.0.0.0/30 — P2P link |
| Border Router | Fa1/0 | Core Switch 2 | Gi1/0/1 | 10.0.0.4/30 — P2P link |
| Core Switch 1 | Gi1/0/3+4 | Core Switch 2 | Gi1/0/3+4 | 10.0.0.8/30 — EtherChannel Po1 (LACP) |
| Core Switch 1 | Gi1/0/2 | Dist Switch 1 | Gi1/0/1 | 10.0.0.16/30 — P2P link |
| Core Switch 1 | Gi1/0/5 | Dist Switch 2 | Gi1/0/2 | 10.0.0.28/30 — P2P link |
| Core Switch 2 | Gi1/0/2 | Dist Switch 2 | Gi1/0/1 | 10.0.0.20/30 — P2P link |
| Core Switch 2 | Gi1/0/5 | Dist Switch 1 | Gi1/0/2 | 10.0.0.24/30 — P2P link |
| Dist Switch 1 | Gi1/0/3 | Access Switch 1 | Gi0/1 | Trunk — VLAN 10, 80, 99 |
| Dist Switch 1 | Gi1/0/4 | Access Switch 2 | Gi0/1 | Trunk — VLAN 20, 99 |
| Dist Switch 1 | Gi1/0/5 | Access Switch 5 | Gi0/1 | Trunk — VLAN 70, 99 |
| Dist Switch 1 | Gi1/0/6 | WLC 3504 | Port 1 | Phase 2: Trunk — VLAN 10,20,30,40,50,70,80,90,99 (was routed port in Phase 1) |
| Dist Switch 2 | Gi1/0/3 | Access Switch 3 | Gi0/1 | Trunk — VLAN 30, 99 |
| Dist Switch 2 | Gi1/0/4 | Access Switch 4 | Gi0/1 | Trunk — VLAN 40, 99 |
| Dist Switch 2 | Gi1/0/5 | Access Switch 6 | Gi0/1 | Trunk — VLAN 60, 99 |
| Dist Switch 2 | Gi1/0/6 | AP-Guests | Gi0 | Access port — VLAN 50 (Guest AP) |

NOTE: Phase 2 change: Dist SW1 Gi1/0/6 was a routed port (192.168.90.1) in Phase 1. In Phase 2 it was reconfigured as a trunk port carrying VLANs 10,20,30,40,50,70,80,90,99 to support the WLC 3504. The IP address moved to Vlan90 SVI.

# 3. Hardware Inventory

| Device | Model | Qty | Role | Est. Cost (USD) |
| --- | --- | --- | --- | --- |
| ISP Router | Cisco 2911 | 1 | Simulates ISP internet gateway (AS 65000) | $1,800 |
| Border Router | Cisco 2811 | 1 | BGP peering, NAT, OSPF edge (AS 65001) | $1,800 |
| Core Switch | Cisco 3650 | 2 | Layer 3 backbone — routing between Distribution and Border | $4,000 |
| Distribution Switch | Cisco 3650 | 2 | Inter-VLAN routing, HSRP gateway, STP root | $4,000 |
| Access Switch | Cisco 3650 | 6 | End-device connectivity and VLAN access port assignment | $1,500 |
| Wireless LAN Controller | Cisco WLC 3504 | 1 | Centralized management of all APs via CAPWAP; 802.1X/WPA2-Enterprise (Phase 2) | $3,500 |
| Access Points | Cisco AP (lightweight) | 7 | One AP per VLAN (VLANs 10,20,30,40,50,70,80) — static IP .6 in each VLAN | $600 |
| RADIUS/DNS Server | Server-PT | 1 | 192.168.60.11 — Dual role: DNS resolution + 802.1X RADIUS authentication (Phase 2) | — |
| DHCP Server | Server-PT | 1 | 192.168.60.12 — Centralized DHCP for all VLANs | — |
| File Server | Server-PT | 1 | 192.168.60.10 — Shared file storage | — |

NOTE: Phase 2 update: The DNS Server at 192.168.60.11 was also configured as the RADIUS server for 802.1X authentication. DHCP server moved from 192.168.60.2 (Phase 1 design) to 192.168.60.12 (Phase 2 implementation). IP addressing for VLAN 60 revised accordingly.

# 4. VLAN Design & IP Addressing

## 4.1 VLAN Segmentation

| VLAN | Name | Department | Subnet | Mask | HSRP VIP (GW) | Usable Hosts |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Operations | Port ops staff | 192.168.10.0/27 | 255.255.255.224 | 192.168.10.30 | 30 |
| 20 | Finance | Finance dept | 192.168.20.0/28 | 255.255.255.240 | 192.168.20.14 | 14 |
| 30 | Management | Management staff | 192.168.30.0/28 | 255.255.255.240 | 192.168.30.14 | 14 |
| 40 | IT | IT and security | 192.168.40.0/28 | 255.255.255.240 | 192.168.40.14 | 14 |
| 50 | Guests | Visitor wireless | 192.168.50.0/22 | 255.255.252.0 | 192.168.51.254 | 1022 |
| 60 | Servers | DHCP, DNS, File | 192.168.60.0/28 | 255.255.255.240 | 192.168.60.14 | 14 |
| 70 | Surveillance | CCTV cameras | 192.168.70.0/27 | 255.255.255.224 | 192.168.70.30 | 30 |
| 80 | Cargo | Cargo operations | 192.168.80.0/28 | 255.255.255.240 | 192.168.80.14 | 14 |
| 90 | WLC-Management | WLC 3504 mgmt | 192.168.90.0/24 | 255.255.255.0 | 192.168.90.1 | 254 |
| 99 | Native | Trunk native VLAN | N/A | N/A | N/A | N/A |

## 4.2 VLAN 60 — Server Static IP Assignments (Updated)

| Device | IP Address | Subnet Mask | Default GW | Role |
| --- | --- | --- | --- | --- |
| File Server | 192.168.60.10 | 255.255.255.240 | 192.168.60.14 | Shared file storage |
| DNS + RADIUS Server | 192.168.60.11 | 255.255.255.240 | 192.168.60.14 | DNS resolution |
| DHCP Server | 192.168.60.12 | 255.255.255.240 | 192.168.60.14 | DHCP service for all VLANs |

## 4.3 WLC Management Network (Phase 2)

| Device | IP Address | Subnet Mask | Default GW | Connected To |
| --- | --- | --- | --- | --- |
| WLC 3504 | 192.168.90.2 | 255.255.255.0 | 192.168.90.1 | Dist SW1 Gi1/0/6 (trunk, native VLAN 99) |
| Dist SW1 Vlan90 SVI | 192.168.90.1 | 255.255.255.0 | N/A | Gateway for WLC management network (Phase 2 SVI) |

## 4.5 Point-to-Point Routing Links (/30 Subnets)

| Link | Device A | Port A | Device B | Port B | Network |
| --- | --- | --- | --- | --- | --- |
| ISP — Border | ISP Router | Gi0/0 | Border Router | Gi0/2 | 203.0.113.0/30 |
| Border — Core1 | Border Router | Gi0/0 | Core Switch 1 | Gi1/0/1 | 10.0.0.0/30 |
| Border — Core2 | Border Router | Gi0/1 | Core Switch 2 | Gi1/0/1 | 10.0.0.4/30 |
| Core1 — Core2 (EtherCh) | Core Switch 1 | Gi1/0/3+4 | Core Switch 2 | Gi1/0/3+4 | 10.0.0.8/30 (Port-Channel 1) |
| Core1 — Dist1 | Core Switch 1 | Gi1/0/2 | Dist Switch 1 | Gi1/0/1 | 10.0.0.16/30 |
| Core1 — Dist2 | Core Switch 1 | Gi1/0/5 | Dist Switch 2 | Gi1/0/2 | 10.0.0.28/30 |
| Core2 — Dist2 | Core Switch 2 | Gi1/0/2 | Dist Switch 2 | Gi1/0/1 | 10.0.0.20/30 |
| Core2 — Dist1 | Core Switch 2 | Gi1/0/5 | Dist Switch 1 | Gi1/0/2 | 10.0.0.24/30 |

5. Redundancy & High Availability

## 5.1 EtherChannel — Core Switch Redundancy

Core Switch 1 and Core Switch 2 are interconnected via two parallel GigabitEthernet links (Gi1/0/3 and Gi1/0/4) bundled into a logical EtherChannel using LACP (IEEE 802.3ad) in active/active mode. This provides link redundancy, bandwidth aggregation, and a single Layer 3 Port-Channel interface for OSPF routing.

## 5.2 HSRP — Default Gateway Redundancy

HSRPv2 is configured on both Distribution switches for every VLAN SVI. Each VLAN uses the last usable IP as the HSRP VIP. Preempt is enabled so the higher-priority switch automatically reclaims the Active role after recovery.

| VLAN Group | HSRP Active | HSRP Standby | Priority Active | Priority Standby | Virtual IP |
| --- | --- | --- | --- | --- | --- |
| VLANs 10, 20, 70, 80 | Dist Switch 1 | Dist Switch 2 | 110 | 90 | .last per subnet |
| VLANs 30, 40, 50, 60 | Dist Switch 2 | Dist Switch 1 | 110 | 90 | .last per subnet |

## 5.3 Rapid PVST+ — Spanning Tree Load Balancing

Rapid PVST+ is configured across all switches. STP root placement is aligned with HSRP Active to ensure traffic takes the optimal path.

| VLAN Group | STP Root Primary | STP Root Secondary | Priority Primary | Priority Secondary |
| --- | --- | --- | --- | --- |
| VLANs 10, 20, 70, 80 | Dist Switch 1 | Dist Switch 2 | 4096 | 8192 |
| VLANs 30, 40, 50, 60 | Dist Switch 2 | Dist Switch 1 | 4096 | 8192 |
| Core Switches (all VLANs) | Never root | Never root | 16384 | 16384 |
| Access Switches (all VLANs) | Never root | Never root | 32768 (default) | 32768 (default) |

6. OSPF Internal Routing Design

## 6.1 Protocol Selection and Design Rationale

Open Shortest Path First (OSPF) was selected as the internal routing protocol for the KPA enterprise network. OSPF is a link-state protocol that builds a complete map of the network topology and calculates the shortest path to every destination. It was chosen over alternatives such as EIGRP due to its vendor-neutral, standards-based nature (RFC 2328), its fast convergence behaviour, and its suitability for multi-vendor enterprise environments.

A single OSPF Area 0 (the backbone area) is used across the entire internal network. This flat single-area design is appropriate for the size of the KPA network and simplifies routing table management, troubleshooting, and future expansion. All five Layer 3 devices participate in OSPF: the Border Router, Core Switch 1, Core Switch 2, Distribution Switch 1, and Distribution Switch 2.

## 6.2 Router ID Assignment

Each routing device is assigned a unique, stable loopback interface that serves as its OSPF Router ID. Loopback interfaces are always up as long as the device is operational, ensuring that the Router ID never changes due to a physical interface failure. This guarantees stable neighbour relationships and prevents unnecessary OSPF reconvergence. The loopback addresses also participate in OSPF so that any device in the network can reach any other device by its loopback IP regardless of physical path.

| Device | Loopback IP | OSPF Router ID | BGP AS (where applicable) |
| --- | --- | --- | --- |
| Border Router | 2.2.2.2/32 | 2.2.2.2 | AS 65001 |
| Core Switch 1 | 3.3.3.3/32 | 3.3.3.3 | N/A |
| Core Switch 2 | 4.4.4.4/32 | 4.4.4.4 | N/A |
| Distribution Switch 1 | 5.5.5.5/32 | 5.5.5.5 | N/A |
| Distribution Switch 2 | 6.6.6.6/32 | 6.6.6.6 | N/A |

## 6.3 Network Advertisements per Device

Each device advertises only the networks directly connected to it into OSPF Area 0. Point-to-point /30 routed links are advertised by both endpoints of the link. VLAN subnets are advertised exclusively by the Distribution switches, which own the SVIs for all user-facing networks. The ISP-facing link (203.0.113.0/30) on the Border Router is deliberately excluded from OSPF — it is managed solely by BGP and must not be visible to internal devices.

| Device | Networks Advertised into OSPF Area 0 |
| --- | --- |
| Border Router | Loopback 2.2.2.2/32 ¦ 10.0.0.0/30 (link to Core SW1) ¦ 10.0.0.4/30 (link to Core SW2) |
| Core Switch 1 | Loopback 3.3.3.3/32 ¦ 10.0.0.0/30 (to Border) ¦ 10.0.0.8/30 (EtherChannel to Core SW2) ¦ 10.0.0.16/30 (to Dist SW1) ¦ 10.0.0.28/30 (to Dist SW2) |
| Core Switch 2 | Loopback 4.4.4.4/32 ¦ 10.0.0.4/30 (to Border) ¦ 10.0.0.8/30 (EtherChannel to Core SW1) ¦ 10.0.0.20/30 (to Dist SW2) ¦ 10.0.0.24/30 (to Dist SW1) |
| Distribution Switch 1 | Loopback 5.5.5.5/32 ¦ 10.0.0.16/30 & 10.0.0.24/30 (uplinks to Core) ¦ VLAN SVIs: 192.168.10.0/27, /28 x4, /22, /27, /28 ¦ VLAN 90: 192.168.90.0/24 (added Phase 2) |
| Distribution Switch 2 | Loopback 6.6.6.6/32 ¦ 10.0.0.20/30 & 10.0.0.28/30 (uplinks to Core) ¦ VLAN SVIs: same ranges as Dist SW1 ¦ No VLAN 90 — WLC management SVI exists on Dist SW1 only |

## 6.4 Passive Interfaces

OSPF passive interfaces are configured on all VLAN SVIs and loopback interfaces on the Distribution switches and Border Router. A passive interface continues to advertise its subnet into OSPF but does not send or process OSPF Hello packets on that interface. This prevents OSPF neighbour relationships from being attempted with end devices such as workstations, servers, cameras, and access points, which could otherwise receive unwanted OSPF traffic. Only the point-to-point routed links between infrastructure devices actively exchange OSPF hellos and form adjacencies.

## 6.5 Point-to-Point Network Type

On all /30 inter-device links, the OSPF network type is explicitly set to point-to-point. By default, OSPF on Ethernet segments operates in broadcast mode and conducts a Designated Router (DR) and Backup Designated Router (BDR) election. Since every inter-device link in this design connects exactly two routing devices, a DR/BDR election serves no purpose and introduces unnecessary delay during OSPF initialisation. Setting the network type to point-to-point eliminates this election, reduces convergence time, and results in a cleaner OSPF neighbour state.

## 6.6 Default Route Distribution

The Border Router is configured to always originate a default route (0.0.0.0/0) into OSPF Area 0, regardless of whether its own routing table contains a default route. This ensures that all internal devices — Core switches, Distribution switches, and by extension all end devices — always have a path toward the internet via the Border Router. In Phase 2, the default route received from the ISP via BGP further reinforces this, with the Border Router redistributing its BGP-learned routes into OSPF so that internal devices can reach external destinations through the established BGP peering session.

7. DHCP Server Configuration (Phase 2)

## 7.1 DHCP Pool Summary

| Pool Name | Network | Subnet Mask | Default GW | DNS | Start IP | End IP |
| --- | --- | --- | --- | --- | --- | --- |
| VLAN10-Operations | 192.168.10.0 | 255.255.255.224 | 192.168.10.30 | 192.168.60.11 | 192.168.10.4 | 192.168.10.29 |
| VLAN20-Finance | 192.168.20.0 | 255.255.255.240 | 192.168.20.14 | 192.168.60.11 | 192.168.20.4 | 192.168.20.13 |
| VLAN30-Management | 192.168.30.0 | 255.255.255.240 | 192.168.30.14 | 192.168.60.11 | 192.168.30.4 | 192.168.30.13 |
| VLAN40-IT | 192.168.40.0 | 255.255.255.240 | 192.168.40.14 | 192.168.60.11 | 192.168.40.4 | 192.168.40.13 |
| VLAN50-Guests | 192.168.50.0 | 255.255.252.0 | 192.168.51.254 | 192.168.60.11 | 192.168.50.4 | 192.168.53.254 |
| VLAN70-Surveillance | 192.168.70.0 | 255.255.255.224 | 192.168.70.30 | 192.168.60.11 | 192.168.70.4 | 192.168.70.29 |
| VLAN80-Cargo | 192.168.80.0 | 255.255.255.240 | 192.168.80.14 | 192.168.60.11 | 192.168.80.4 | 192.168.80.13 |
| VLAN90-WLC-Management | 192.168.90.0 | 255.255.255.0 | 192.168.90.1 | 192.168.60.11 | 192.168.90.4 | 192.168.90.253 |

## 7.2 DHCP Relay — ip helper-address

NOTE: ip helper-address forwards DHCP broadcasts from each VLAN SVI to the DHCP server at 192.168.60.12. Must be configured on BOTH Distribution switches on every VLAN SVI including Vlan90 (Dist SW1 only). Without this, hosts will not receive DHCP responses.

### Distribution Switch 1 — DHCP Relay on All SVIs

interface Vlan10

ip helper-address 192.168.60.12

interface Vlan20

ip helper-address 192.168.60.12

interface Vlan30

ip helper-address 192.168.60.12

interface Vlan40

ip helper-address 192.168.60.12

interface Vlan50

ip helper-address 192.168.60.12

interface Vlan60

ip helper-address 192.168.60.12

interface Vlan70

ip helper-address 192.168.60.12

interface Vlan80

ip helper-address 192.168.60.12

interface Vlan90

ip helper-address 192.168.60.12

### Distribution Switch 2 — DHCP Relay on All SVIs

interface Vlan10

ip helper-address 192.168.60.12

interface Vlan20

ip helper-address 192.168.60.12

interface Vlan30

ip helper-address 192.168.60.12

interface Vlan40

ip helper-address 192.168.60.12

interface Vlan50

ip helper-address 192.168.60.12

interface Vlan60

ip helper-address 192.168.60.12

interface Vlan70

ip helper-address 192.168.60.12

interface Vlan80

ip helper-address 192.168.60.12

! NOTE: Dist SW2 does NOT have Vlan90 — WLC management SVI is on Dist SW1 only.

# 8. PortFast & BPDUGuard (Phase 2)

NOTE: PortFast skips STP Listening/Learning on access ports — ports come up immediately for end devices. BPDUGuard err-disables the port if a BPDU is received, protecting against rogue switches. Applied on all access ports across Access Switches 1–6 and AP/device ports on Distribution switches.

## 8.1 Access Switches 1–6

! Apply on all Access Switches (1 through 6)

interface range FastEthernet0/1 - 24

spanning-tree portfast

spanning-tree bpduguard enable

Note for Access Switch 1: Fa0/1–Fa0/12 = VLAN 80 (Cargo) | Fa0/13–Fa0/24 = VLAN 10 (Operations). AP-Cargo connects on Fa0/1, AP-Operations connects on Fa0/13.

## 8.2 Distribution Switch 2 — Gi1/0/6 (AP-Guests, VLAN 50)

interface GigabitEthernet1/0/6

spanning-tree portfast

spanning-tree bpduguard enable

! switchport mode access vlan 50 already set in Phase 1

9. BGP Internet Connectivity Design (Phase 2)

## 9.1 Protocol Selection and Design Rationale

External Border Gateway Protocol (eBGP) is used to establish internet connectivity between the KPA network and the simulated Internet Service Provider (ISP). BGP was selected for this role because it is the standard inter-domain routing protocol used universally between organisations and ISPs. Unlike OSPF, which is designed for routing within a single organisation, BGP is purpose-built for controlled route exchange between separate administrative domains (Autonomous Systems).

The KPA network operates as Autonomous System 65001, while the ISP operates as Autonomous System 65000. AS numbers in the 65000–65535 range are private AS numbers reserved for internal or simulated use, consistent with best practice in lab and production environments that do not connect to the global internet.

## 9.2 BGP Peering Session

A single eBGP peering session is established between the Border Router and the ISP Router across the 203.0.113.0/30 point-to-point link. This link is intentionally excluded from OSPF — it is a demarcation link between the two autonomous systems and should only be known to the two directly connected devices. The Border Router uses its loopback address (2.2.2.2) as the BGP Router ID for consistency with its OSPF identity.

| Parameter | Border Router (KPA) | ISP Router | Notes |
| --- | --- | --- | --- |
| Autonomous System | AS 65001 | AS 65000 | Private AS numbers |
| BGP Router ID | 2.2.2.2 | 1.1.1.1 | Matches OSPF loopback |
| Peering Interface | Gi0/2 — 203.0.113.2 | Gi0/0 — 203.0.113.1 | Not in OSPF |
| Peering Link | 203.0.113.0/30 | 203.0.113.0/30 | ISP demarcation link |

## 9.3 Route Advertisement Strategy

The BGP peering session serves two distinct purposes, one for each direction of traffic flow. Outbound from KPA to the ISP, the Border Router redistributes all internally learned OSPF routes into BGP. This allows the ISP to have full reachability to every KPA subnet — a requirement in scenarios where the ISP or external partners need to initiate connections into the KPA network, such as for remote management or partner integrations. A metric of 100 is applied to redistributed routes to give the ISP a consistent MED (Multi-Exit Discriminator) value for these prefixes.

Inbound from the ISP to KPA, the ISP Router advertises a single default route (0.0.0.0/0) into BGP. This default route is received by the Border Router and, through the 'default-information originate always' directive already configured in OSPF, is propagated as a default route throughout the internal OSPF domain. All internal devices therefore automatically use the Border Router as their path to the internet without requiring any additional static routes.

To advertise the default route via BGP, the ISP Router requires a static default route in its own routing table pointing to its loopback interface. This is a standard BGP requirement: a device cannot advertise a network statement it does not itself have in its routing table. The static route to Loopback0 ensures the ISP always has a valid next-hop for the default route advertisement.

## 9.4 Relationship Between BGP and OSPF

It is important to understand that BGP and OSPF operate as complementary but completely separate routing processes on the Border Router. OSPF handles all routing within the KPA organisation. BGP handles only the exchange of routing information between KPA and the ISP. The redistribution between the two protocols is one-directional by design: OSPF routes flow into BGP (so the ISP knows about internal subnets), but BGP routes do not flow back into OSPF directly. Instead, the default route mechanism in OSPF handles inbound internet routing for all internal devices, keeping the internal routing table clean and avoiding the complexity of importing external BGP prefixes into OSPF.

10. WLC & Wireless LAN — Cisco 3504 (Phase 2)

## WLC Initial Setup (Packet Tracer GUI)

1. Connect WLC Port 1 -> Dist SW1 GigabitEthernet1/0/6 (straight-through cable). 2. WLC GUI > CONTROLLER > Interfaces > Management Interface (edit existing): IP Address : 192.168.90.2 Subnet Mask : 255.255.255.0 Default Gateway: 192.168.90.1 VLAN ID : 90 Port : 1 Click Apply.

## WLC Dynamic Interfaces — One Per VLAN

WLC GUI > CONTROLLER > Interfaces > New. Create one per VLAN:

Dynamic Interface — VLAN 10 (Operations)

Interface Name : vlan10-operations VLAN ID : 10 IP Address : 192.168.10.3 (.3 — below AP .6 and DHCP start .7, no conflicts) Subnet Mask : 255.255.255.224 Default Gateway: 192.168.10.30 (HSRP VIP) DHCP Server IP : 192.168.60.12 Port : 1 Click Apply.

Dynamic Interface — VLAN 50 (Guests)

Interface Name : vlan50-guests VLAN ID : 50 IP Address : 192.168.50.3 (.3 — below AP .6 and DHCP start .7, no conflicts) Subnet Mask : 255.255.252.0 Default Gateway: 192.168.51.254 (HSRP VIP) DHCP Server IP : 192.168.60.12 Port : 1 Click Apply.

Dynamic Interface — VLAN 70 (Surveillance)

Interface Name : vlan70-surveillance VLAN ID : 70 IP Address : 192.168.70.3 (.3 — below AP .6 and DHCP start .7, no conflicts) Subnet Mask : 255.255.255.224 Default Gateway: 192.168.70.30 (HSRP VIP) DHCP Server IP : 192.168.60.12 Port : 1 Click Apply.

## WLAN Creation — 3 WLANs

WLC GUI > WLANs > Create New > Go:

WLAN 1 — Operations-WiFi  (VLAN 10 | WPA2-Personal)

Profile Name : Operations-WiFi SSID : Operations-WiFi WLAN ID : 1 Status : Enabled General tab: Interface : vlan10-operations Security tab > Layer 2: Layer 2 Security : WPA + WPA2 WPA2 Policy : Enabled Password: Ops@Pass1

WLAN 5 — Guests-WiFi  (VLAN 50 | Open)

Profile Name : Guests-WiFi SSID : Guests-WiFi WLAN ID : 5 Status : Enabled General tab: Interface : vlan50-guests Security tab > Layer 2: Layer 2 Security : None (open — no password) Click Apply.

WLAN 6 — Surveillance-WiFi  (VLAN 70 | WPA2-Personal)

Profile Name : Surveillance-WiFi SSID : Surveillance-WiFi WLAN ID : 3 Status : Enabled General tab: Interface : vlan70-surveillance Security tab > Layer 2: Layer 2 Security : WPA + WPA2 WPA2 Policy : Enabled Password; Surv@Pass1

## IP Phone Deployment

## This phase implements IP telephony within the enterprise network using a centralized Voice VLAN (VLAN 100), DHCP services, and Cisco Call Manager Express (CME) hosted on the border router. The goal is to enable seamless voice communication across departments while maintaining proper network segmentation.

## 1. Voice VLAN Design (VLAN 100)

### Purpose

A dedicated Voice VLAN (VLAN 100) is used to:

- Isolate voice traffic from data traffic

- Improve QoS and performance

- Simplify IP phone deployment across departments

### Key Characteristics

- VLAN ID: 100

- Network: 172.16.100.0/24

- Default Gateway (HSRP VIP): 172.16.100.254

- Shared across all departments

### Redundancy (HSRP)

- Distribution Switch 1

  - Role: Active

  - Priority: 110

- Distribution Switch 2

  - Role: Standby

  - Priority: 100

- Failover Behavior

  - If Dist SW1 fails → Dist SW2 automatically becomes gateway

## 3. IP Phone Mapping Table

| Phone Name | Department | Data VLAN | Voice VLAN | Access Switch | Port | IP Range | Extension |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Phone-Operations | Operations | VLAN 10 | VLAN 100 | Access SW1 | Fa0/2 | 172.16.100.x | 1001 |
| Phone-Cargo | Cargo | VLAN 80 | VLAN 100 | Access SW1 | Fa0/14 | 172.16.100.x | 1002 |
| Phone-Finance | Finance | VLAN 20 | VLAN 100 | Access SW2 | Fa0/3 | 172.16.100.x | 1003 |
| Phone-Management | Management | VLAN 30 | VLAN 100 | Access SW3 | Fa0/3 | 172.16.100.x | 1004 |
| Phone-IT | IT | VLAN 40 | VLAN 100 | Access SW4 | Fa0/3 | 172.16.100.x | 1005 |

## 4. DHCP Configuration (Voice VLAN 100)

### DHCP Server Details

- Server IP: 192.168.60.12

- Scope Name: VLAN100-Voice

### Addressing Scheme

- Network: 172.16.100.0/24

- Usable Range: 172.16.100.4 – 172.16.100.253

- Excluded Range: 172.16.100.1 – 172.16.100.6

### Options Provided

- Default Gateway: 172.16.100.254

- DNS Server: 192.168.60.11

- TFTP Server (Option 150): 2.2.2.2

### Important Note

The TFTP Server (2.2.2.2) is critical because:

- It points phones to the CME router

- Enables phone configuration download

- Without it → phones fail to register

## 5. DHCP Relay (IP Helper Address)

### Purpose

IP phones send DHCP requests as broadcasts, which cannot cross VLANs.
Distribution switches act as intermediaries to forward these requests.

### Implementation

- Configured on VLAN 100 interface (SVI) on both distribution switches

- Ensures DHCP requests reach the centralized server

### Redundancy Consideration

Both switches must have DHCP relay configured because:

- HSRP may shift gateway responsibility at any time

## 6. Routing Integration (OSPF)

### Objective

Ensure full network reachability between:

- IP phones

- CME router

- Other network segments

### Implementation

- VLAN 100 subnet is advertised into OSPF Area 0

- VLAN interface is set as passive

### Reason for Passive Interface

- Prevents OSPF hello packets from reaching end devices (phones)

- Still allows subnet advertisement

## 7. Access Layer Voice Configuration

### Design Behavior

Each access port connected to an IP phone:

- Supports both data and voice traffic

- Automatically separates traffic using VLAN tagging

### Switch Behavior

- Phone traffic → Voice VLAN 100

- PC traffic → Department VLAN

### Security & Optimization

- PortFast enabled → faster device startup

- BPDU Guard → prevents loops from end devices

## 8. Cisco Call Manager Express (CME)

### Role

The border router functions as:

- Call control system

- TFTP server

- Phone registration manager

### Core Functions

- Assigns extensions (Directory Numbers)

- Registers IP phones

- Manages call routing

### CME Addressing

- Source Address: 2.2.2.2 (Loopback0)

### Directory Numbers

| Extension | Department |
| --- | --- |
| 1001 | Operations |
| 1002 | Cargo |
| 1003 | Finance |
| 1004 | Management |
| 1005 | IT |

## 9. IP Phone Boot & Registration Process

### Step-by-Step Flow

- Phone powers on

- Receives VLAN 100 via switch

- Sends DHCP request

- Distribution switch relays request

- DHCP server assigns IP + TFTP server

- Phone contacts CME (2.2.2.2)

- Downloads configuration

- Registers and becomes operational

## Security Hardening

### Overview

Phase 3 focuses on strengthening the security posture of the enterprise network by implementing device identification, access control, and secure management protocols. This includes hostname standardization, port security at the access layer, secure authentication mechanisms, login banners, and encrypted remote access using SSH. These measures ensure that only authorized users and devices can access the network while protecting against unauthorized connections and attacks.

## 1. Hostname Configuration

To improve network management and device identification, each core and distribution switch is assigned a unique hostname that reflects its role within the network. This naming convention simplifies troubleshooting, monitoring, and administrative tasks across the infrastructure.

### Hostname Assignment Table

| Device | Hostname |
| --- | --- |
| Core Switch 1 | ADMIN |
| Core Switch 2 | MANAGER |
| Distribution Switch 1 | ANALYST |
| Distribution Switch 2 | DESIGNER |

## 2. Port Security Implementation

Port security is enforced across all access switches to control device connectivity and prevent unauthorized access. Each access port is configured to allow only a single device using sticky MAC address learning, meaning the first connected device’s MAC address is dynamically learned and secured.

If a second device attempts to connect to the same port, the switch automatically places the port into an error-disabled (shutdown) state, effectively blocking unauthorized access. This mechanism is particularly important in preventing attacks such as MAC flooding and unauthorized endpoint connections.

### Port Security Policy Summary

| Feature | Configuration Applied |
| --- | --- |
| Maximum MAC Addresses | 1 per port |
| MAC Address Learning | Sticky |
| Violation Mode | Shutdown |
| Ports Affected | All access switch ports (Fa0/1–Fa0/24) |

## 3. Banner Message of the Day (MOTD)

A standardized Message of the Day (MOTD) banner is configured across all routers and switches in the network. This banner serves as a legal and security notice to all users attempting to access the devices, warning against unauthorized usage.

The banner message used is:

#####Welcome to KPA#####

This ensures consistency and reinforces organizational security policies across all network devices.

## 4. Password and Access Control Configuration

To secure device access, multiple layers of password protection are implemented. The enable secret password is used to secure privileged EXEC mode and is stored in encrypted form. Console access is protected to prevent unauthorized physical access, while remote access via VTY lines is restricted to SSH only.

Additionally, all plaintext passwords in the configuration are encrypted using the built-in password encryption service to prevent exposure.

### Credential Summary Table

| Access Type | Username | Password |
| --- | --- | --- |
| Enable Secret | — | Kpa123 |
| Console Access | — | Kpa1234 |
| SSH Access | admin | Device-specific |

## 5. Secure Remote Access (SSH)

Secure Shell (SSH) is configured on all routers and switches to provide encrypted remote management access. Telnet is disabled to eliminate the risk of transmitting credentials in plaintext.

Each device is configured with:

- A local user account for authentication

- A domain name required for cryptographic key generation

- RSA keys (1024-bit) for encryption

- SSH version 2 for improved security

Different devices use unique SSH credentials to enhance security and accountability.

### SSH Configuration Summary

| Device | Username | SSH Password |
| --- | --- | --- |
| Core Switch 1 (ADMIN) | admin | kpa@admin |
| Core Switch 2 (MANAGER) | admin | kpa@manager |
| Dist Switch 1 (ANALYST) | admin | !analyst@kpa# |
| Dist Switch 2 (DESIGNER) | admin | !analyst@kpa# |

## 6. Verification and Validation

After implementation, several verification steps are performed to ensure security features are functioning correctly. These include testing port security by attempting unauthorized connections, verifying SSH access to confirm encrypted remote login, and ensuring password protections are enforced across all access points.

Successful validation confirms that:

- Unauthorized devices are blocked at access ports

- Remote access is restricted to secure SSH sessions

- All devices enforce authentication and encryption policies

).